FM94-BUFR.doc

The MEDS FM94-BUFR Data Set System

This document prepared July 2013 by Graham Glenn and Mathieu Ouellet (MEDS)

Marine Environmental Data Service (MEDS)

Integrated Science Data Management (ISDM)

Department of Fisheries and Oceans (DFO)

Government of Canada

Introduction

Global Telecommunications Service (GTS)  Drifting Buoy Messages in FM94-BUFR

Also from the GTS, and since at least 2004, every month a large proportion of the same data as reported in FM18-BUOY is also reported in the same real time schedule, in a BUFR version FM94-BUFR .  The plan of the global coordinators has been to phase in the use of BUFR to replace the FM18-BUOY format. To date, not all FM18-BUOY messages have a corresponding FM94-BUFR message, and this is being investigated by MEDS, IFREMER in France, and the coordinating agency. Also, presently, several buoys are only reporting in FM94-BUFR.

The data collection, and distribution from the buoys to MEDS is the same as described by the FM18_BUOY.doc report. However the data messages are in BUFR.

Acquisition

Automatic Data File Acquisition

Similar to the system for the FM18-BUOY message, JCOMMOPS has supplied MEDS with a list of GTS bulletin headers, which are used to select the FM94-BUFR messages from broadcast meteorological and oceanographic data.

MEDS also provided this list of BUFR bulletin headers to an agency in Environment Canada with access to the GTS. The Environment Canada agency receives the GTS broadcast (from USA), and sends their selected bulletins in files delivered every half hour by FTP. These files contain all of the bulletins of interest with only FM94-BUFR messages.

Decode and Reformat

A description of the contents of the FM94-BUFR format is included at the end of this document.

Decoding the GTS FM94-BUFR message is a scheduled and automated procedure running daily. The acquired files are saved for backup. BUOY messages are read from the GTS acquired file, and converted to MEDS Oceans format.

Quality control of the incoming formatted data is performed by the software, to check for inconsistencies in the format of the message. Messages with BUFR formats inconsistent with FM94-BUFR are ignored by the input stream.

The daily decoded data messages are appended to a holding file.

Reports

MEDS staff have been provided with a application to compare a month of data received in FM94-BUFR to the data received in FM18-BUOY.

Tables

BUFR Drifting Buoy Template

| DESCRIPTOR | DESCRIPTION |
| --- | --- |
| 001003 | WMO Region |
| 001020 | WMO region sub-area |
| 001005 | Buyo/platform identifier |
| 002001 | Type of station |
| 002036 | BUOY type |
| 002149 | Type of data buoy |
| 301011 | Date |
| 301012 | Time |
| 008021 | Time Significance(=26 time of last know position) |
| 301011 | Date |
| 301012 | Time |
| 008021 | Time significance (set to missing to cancel previous value) |
| 301021 | Latitude and Longitude (high accuracy) |
| 027004 | Alternate latitude (high accuracy) |
| 028004 | Alternat longitude (high accuracy) |
| 007030 | Height of station above MSL |
| 001051 | Platform Transmitter ID (CCITT IA5) |
| 002148 | Data collection and/or Location system |
| 001012 | Platform Drift Direction |
| 001014 | Platform Drift Speed |
| 002040 | Method of removing platform direction and speed from current |
| 033022 | Quality of buoy satellite transmission |
| 033023 | Quality of buoy location |
| 033027 | Location quality class (range of radius of 66% confidence) |
| 022063 | Total water depth |
| 302021 | Waves |
| 302022 | Wind waves |
| 302023 | Swell waves |
| 008081 | Type of equipment(observing platform) |
| 025026 | Battery Voltage |
| 008081 | Type of equipment(transmitter) |
| 025026 | Battery Voltage |
| 008081 | Type of equipment(receiver) |
| 025026 | Battery Voltage |
| 008081 | Type of equipment(set to missing to cancel the previous value) |
| 002034 | Drogue type |
| 022060 | Lagrangian drifter drogue status |
| 007070 | Drogue depth |
| 002190 | Lagrangian drifter submergence |
| 025086 | Depth correction indicator |
| 002035 | Cable length |
| 002168 | Hydrostatic pressure of lower end of cable |
| 020031 | Ice deposit (thickness) |
| 002038 | Method of water temperature and/or salinity measurement |
| 306004 | Digitization, depth/salinity method, depths/salinities/temperatures |
| 002030 | Method of current measurement |
| 306005 | Time/duration of current measurement, depths/directions/speeds |
| 007031 | Height of barometer above MSL |
| 008081 | Type of equipment (sensor) |
| 012064 | Instrument temperature |
| 302001 | Pressure and pressure change |
| 008081 | Type of equipment(set to missing to cancel the previous value) |
| 007032 | Height of sensor above marine deck platform (for temp.&hum. measurement) |
| 007033 | Height of sensor above water surface (for temp.&hum. measurement) |
| 012101 | Dry-bulb temperature (scale 2) |
| 012103 | Dew-point temperature (scale 2) |
| 013003 | Relative humidity |
| 007032 | Height of sensor above marine deck platform (for wind measurement) |
| 007033 | Height of sensor above water surface (for wind measurement) |
| 008082 | Modification of sensor height to another value |
| 007033 | Height of sensor above water surface (here height of anemometer to which it is modified) |
| 002169 | Anemometer type |
| 002002 | Type of instrumentation for wind measurement |
| 008021 | Time significance (=2 (time averaged)) |
| 004025 | Time period in minutes |
| 011001 | Wind direction |
| 011002 | Wind speed |
| 008021 | Time significance (set to missing to cancel previous value) |
| 004025 | Time period in minutes" |
| 011043 | Maximum wind gust direction |
| 011041 | Maximum wind gust speed |
| 008082 | Modification of sensor height to another value (set to missing to reset previous value) |
| 007033 | Height of sensor above water surface (set to missing to cancel previous value) |
| 007032 | Height of sensor above marine deck platform (for precipitation measurement) |
| 004024 | Time period in hours |
| 013011 | Total precipitation |
| 007032 | Height of sensor above marine deck platform (set to missing to cancel the previous value) |
| 008021 | Time significance (=3 (accumulated)) |
| 004024 | Time period in hours |
| 014021 | Global radiation integrated over period specified |
| 008021 | Time significance (set to missing to cancel previous value) |
| 025028 | Operator or manufacturer defined parameter (#1 |
| 025028 | Operator or manufacturer defined parameter (#1) |
| 025028 | Operator or manufacturer defined parameter (#1) |

Decode BUFR Fields to MEDS Format

DESCRIPTOR, Descriptor (FXY)

DESCRIPTOR_CHILD, Expanded descriptor (FXY)

FORCED_VALUE, Value that has been forced.

FORCED_MISSING, Determines if value was forced to be missing (Y/N)

MEDS_PCODE, PCODE used by MEDS to describe value

UNITS, Units of the data (units of data in BUFR template may be converted to MEDS PCODE units)

CODE_TABLE_MAX, Maximum value indicated by the code table described by the descriptor

Scale, The power of 10 that the element in Section 4 have been multiplied by in order to retain the desired precision in the transmitted data.

REFERENCE, A value to be subtracted from the data after multiplication by the scale factor before encoding into Section 4 in order to produce a positive value.

DATA_WIDTH, A count of how many bits the largest possible value of an individual data item of Section 4 occupies.

| 001003 |  |  |  | WMOR | codetable | 7 | 0 | 0 | 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 001020 |  |  |  | WMOS | numeric | 0 | 0 | 0 | 4 |
| 001005 |  |  |  | CR_NUMBER | numeric | 0 | 0 | 0 | 17 |
| 002001 |  |  |  | IGNORE | codetable | 3 | 0 | 0 | 2 |
| 002036 |  |  |  | BUOY | codetable | 3 | 0 | 0 | 2 |
| 002149 |  |  |  | BTP$ | codetable | 63 | 0 | 0 | 6 |
| 301011 | 004001 |  |  | OBS_YEAR | numeric | 0 | 0 | 0 | 12 |
| 301011 | 004002 |  |  | OBS_MONTH | numeric | 0 | 0 | 0 | 4 |
| 301011 | 004003 |  |  | OBS_DAY | numeric | 0 | 0 | 0 | 6 |
| 301012 | 004004 |  |  | OBS_HOUR | numeric | 0 | 0 | 0 | 5 |
| 301012 | 004005 |  |  | OBS_MINUTE | numeric | 0 | 0 | 0 | 6 |
| 008021 |  | 26 |  | NONE | codetable | 31 | 0 | 0 | 5 |
| 301011 | 004001 |  |  | PDTY | numeric | 0 | 0 | 0 | 12 |
| 301011 | 004002 |  |  | PDTM | numeric | 0 | 0 | 0 | 4 |
| 301011 | 004003 |  |  | PDTD | numeric | 0 | 0 | 0 | 6 |
| 301012 | 004004 |  |  | PTMH | numeric | 0 | 0 | 0 | 5 |
| 301012 | 004005 |  |  | PTMM | numeric | 0 | 0 | 0 | 6 |
| 008021 |  |  | Y | NONE | codetable | 31 | 0 | 0 | 5 |
| 301021 | 005001 |  |  | LATITUDE | degree | 0 | 5 | -9000000 | 25 |
| 301021 | 006001 |  |  | LONGITUDE | degree | 0 | 5 | -18000000 | 26 |
| 027004 |  |  |  | LTG$ | degree | 0 | 5 | -9000000 | 25 |
| 028004 |  |  |  | LNG$ | degree | 0 | 5 | -18000000 | 26 |
| 007030 |  |  |  | ALTS | m | 0 | 1 | -4000 | 17 |
| 001051 |  |  |  | PTT$ | char | 0 | 0 | 0 | 96 |
| 002148 |  |  |  | SYS$ | codetable | 31 | 0 | 0 | 5 |
| 001012 |  |  |  | DDIR | degreetrue | 0 | 0 | 0 | 9 |
| 001014 |  |  |  | DSPD | m/s | 0 | 2 | 0 | 10 |
| 002040 |  |  |  | GGK6 | codetable | 15 | 0 | 0 | 4 |
| 033022 |  |  |  | BQN$ | codetable | 3 | 0 | 0 | 2 |
| 033023 |  |  |  | BQL$ | codetable | 3 | 0 | 0 | 2 |
| 033027 |  |  |  | BQA$ | codetable | 7 | 0 | 0 | 3 |
| 022063 |  |  |  | BATH | m | 0 | 0 | 0 | 14 |
| 302021 | 022001 |  |  | VPED | degreetrue | 0 | 0 | 0 | 9 |
| 302021 | 022011 |  |  | VTPK | s | 0 | 0 | 0 | 6 |
| 302021 | 022021 |  |  | VCAR | m | 0 | 1 | 0 | 10 |
| 302022 | 022002 |  |  | WDR$ | degreetrue | 0 | 0 | 0 | 9 |
| 302022 | 022012 |  |  | WPR$ | s | 0 | 0 | 0 | 6 |
| 302022 | 022022 |  |  | WHT$ | m | 0 | 1 | 0 | 10 |
| 302023 | 022003 |  |  | SWDR | degreetrue | 0 | 0 | 0 | 9 |
| 302023 | 022013 |  |  | SWPR | s | 0 | 0 | 0 | 6 |
| 302023 | 022023 |  |  | SWHT | m | 0 | 1 | 0 | 10 |
| 008081 |  | 3 |  | NONE | codetable | 63 | 0 | 0 | 6 |
| 025026 |  |  |  | BAVP | V | 0 | 1 | 0 | 12 |
| 008081 |  | 1 |  | NONE | codetable | 63 | 0 | 0 | 6 |
| 025026 |  |  |  | BAVT | V | 0 | 1 | 0 | 12 |
| 008081 |  | 2 |  | NONE | codetable | 63 | 0 | 0 | 6 |
| 025026 |  |  |  | BAVR | V | 0 | 1 | 0 | 12 |
| 008081 |  |  | Y | NONE | codetable | 63 | 0 | 0 | 6 |
| 002034 |  |  |  | DRT$ | codetable | 31 | 0 | 0 | 5 |
| 022060 |  |  |  | DRST | codetable | 7 | 0 | 0 | 3 |
| 007070 |  |  |  | DRDP | m | 0 | 0 | 0 | 10 |
| 002190 |  |  |  | LDS$ | % | 0 | 0 | 0 | 7 |
| 025086 |  |  |  | BQZ$ | codetable | 3 | 0 | 0 | 2 |
| 002035 |  |  |  | CLN$ | m | 0 | 0 | 0 | 9 |
| 002168 |  |  |  | PRC$ | Pa | 0 | -3 | 0 | 16 |
| 020031 |  |  |  | ICT$ | m | 0 | 2 | 0 | 7 |
| 002038 |  |  |  | IST$ | codetable | 15 | 0 | 0 | 4 |
| 306004 | 002032 |  |  | GGDI | codetable | 3 | 0 | 0 | 2 |
| 306004 | 002033 |  |  | GGSL | codetable | 7 | 0 | 0 | 3 |
| 306004 | 103000 |  |  | NONE | none | 0 | 0 | 0 | 0 |
| 306004 | 031001 |  |  | NONE | numeric | 0 | 0 | 0 | 8 |
| 306004 | 007062 |  |  | DEPH | m | 0 | 1 | 0 | 17 |
| 306004 | 022043 |  |  | TEMP | K | 0 | 2 | 0 | 15 |
| 306004 | 022062 |  |  | PSAL | part/thou | 0 | 2 | 0 | 14 |
| 002030 |  |  |  | GGCM | codetable | 7 | 0 | 0 | 3 |
| 306005 | 002031 |  |  | GGEC | codetable | 31 | 0 | 0 | 5 |
| 306005 | 103000 |  |  | NONE | none | 0 | 0 | 0 | 0 |
| 306005 | 031001 |  |  | NONE | numeric | 0 | 0 | 0 | 8 |
| 306005 | 007062 |  |  | DEPH | m | 0 | 1 | 0 | 17 |
| 306005 | 022004 |  |  | HCDT | degreetrue | 0 | 0 | 0 | 9 |
| 306005 | 022031 |  |  | HCSP | m/s | 0 | 2 | 0 | 13 |
| 007031 |  |  |  | BAH$ | m | 0 | 1 | -4000 | 17 |
| 008081 |  | 0 |  | NONE | codetable | 63 | 0 | 0 | 6 |
| 012064 |  |  |  | BART | K | 0 | 1 | 0 | 12 |
| 302001 | 010004 |  |  | ATMP | Pa | 0 | -1 | 0 | 14 |
| 302001 | 010051 |  |  | ATMS | Pa | 0 | -1 | 0 | 14 |
| 302001 | 010061 |  |  | ATPT | Pa | 0 | -1 | -500 | 10 |
| 302001 | 010063 |  |  | GPT$ | codetable | 15 | 0 | 0 | 4 |
| 008081 |  |  | Y | NONE | codetable | 63 | 0 | 0 | 6 |
| 007032 |  |  |  | THT$ | m | 0 | 2 | 0 | 16 |
| 007033 |  |  |  | THTS | m | 0 | 1 | 0 | 12 |
| 012101 |  |  |  | DRYT | K | 0 | 2 | 0 | 16 |
| 012103 |  |  |  | DEWT | K | 0 | 2 | 0 | 16 |
| 013003 |  |  |  | RELH | % | 0 | 0 | 0 | 7 |
| 007032 |  |  |  | ANH$ | m | 0 | 2 | 0 | 16 |
| 007033 |  |  |  | ANHS | m | 0 | 1 | 0 | 12 |
| 008082 |  |  |  | AHTA | codetable | 7 | 0 | 0 | 3 |
| 007033 |  |  |  | ANHA | m | 0 | 1 | 0 | 12 |
| 002169 |  |  |  | ANT$ | codetable | 15 | 0 | 0 | 4 |
| 002002 |  |  |  | IGNORE | flagtable | 0 | 0 | 0 | 4 |
| 008021 |  | 2 |  | NONE | codetable | 31 | 0 | 0 | 5 |
| 004025 |  |  |  | WTMM | minute | 0 | 0 | -2048 | 12 |
| 011001 |  |  |  | WDIR | degreetrue | 0 | 0 | 0 | 9 |
| 011002 |  |  |  | WSPD | m/s | 0 | 1 | 0 | 12 |
| 008021 |  |  | Y | NONE | codetable | 31 | 0 | 0 | 5 |
| 004025 |  |  |  | GTMM | minute | 0 | 0 | -2048 | 12 |
| 011043 |  |  |  | GDIR | degreetrue | 0 | 0 | 0 | 9 |
| 011041 |  |  |  | GSPD | m/s | 0 | 1 | 0 | 12 |
| 008082 |  |  | Y | NONE | codetable | 7 | 0 | 0 | 3 |
| 007033 |  |  | Y | NONE | m | 0 | 1 | 0 | 12 |
| 007032 |  |  |  | PRHT | m | 0 | 2 | 0 | 16 |
| 004024 |  |  |  | PRTH | hour | 0 | 0 | -2048 | 12 |
| 013011 |  |  |  | PRTN | kg/m2 | 0 | 1 | -1 | 14 |
| 007032 |  |  | Y | NONE | m | 0 | 2 | 0 | 16 |
| 008021 |  | 3 |  | NONE | codetable | 31 | 0 | 0 | 5 |
| 004024 |  |  |  | RTMH | hour | 0 | 0 | -2048 | 12 |
| 014021 |  |  |  | GRAD | J/m2 | 0 | -4 | 0 | 15 |
| 008021 |  |  | Y | NONE | codetable | 31 | 0 | 0 | 5 |
| 025028 |  |  |  | IGNORE | numeric | 0 | 1 | -16384 | 15 |
| 025028 |  |  |  | IGNORE | numeric | 0 | 1 | -16384 | 15 |
| 025028 |  |  |  | IGNORE | numeric | 0 | 1 | -16384 | 15 |