Converting FORM 700 Series to LMR

Form Types 701, 702, 703, 720, & 721

No. Field  Element	Description         Procedure

No.

==============================================================================

RPTIN

RPTID

1  B10	       10-deg box  		Compute based on Lat/Long; translate to LMR

2  YR     800	  year        		Translate to LMR

3  MO     696	  month             Convert local to GMT; translate to LMR

4  DY     695	  day               Convert local to GMT; translate to LMR

5  HR     	  hour               Convert local ship time or 75th merdian         	 	/690  (local ship time)	time to GMT; translate to LMR

/691  (75th merdian)

6  TI     	  time indic.       Set to 0; nearest whole hour (GMT)

7  LON    751   longitude	     Longitude is in degree & minutes; hemisphere

/802   (hemisphere)		is reported as E or W except when an 								estimated	position Then: E = D; W = V: 								convert to LMR convention (0-359.99 degrees 							east). If	estimated position set LI 								(lat/long indic.) = 3 (interpolated 								position) in LMR format.

8  LAT	750    Latitude	     latitude is in degree & minutes; hemisphere

/801    (hemisphere)		is reported as N or S except when an 								estimated position Then: N = M; S = R: 								convert to LMR convention (-90.00 to 90.00; 							90 south to 90 north). If estimated position 							set LI (lat/long indic.) = 3 (interpolated 							position) in LMR format.

9  LI     	  lat/lon indic.    Set to 4; if degrees and miniutes

(reported position)

Set to 3; if position estimated or 							     interpolated

10  DCK    	  deck               Set deck to 700; 1912-1946 U.S Merchant                                          Marine

11  SID    	  source ID          Set ID = 56; (NCDC keyed data)

12  PT     	  platform type      Set to 5; ship

13  QI     	  quality indic.     Left blank; assign as missing

14  DS     	  dup status         Left blank; assign as missing

15  DC     	  dup check          Left blank; assign as missing

16  TC     	  track check        Left blank; assign as missing

17  PB     	  pressure bias      Left blank; assign as missing

Note: need to design a consistency check to        							 to compare pressure corrected (inches)

against barometer as coded (millibars)

18  DI     	  wind dir. indic.   Set to 1; 32 point compass

Note: based on the reports it appears

that many observers were using a more

course resolution (e.g. 16 pt. or even

8 pt. resolution)

19  D      612	  wind direction     Code according to table 8, LMR 						                documentation

Note 1: code values range from 0-32 plus

37 = variable and 39 = dash (-).

Note 2: after the conversions were turned                                        off the directions may appear as alpha 							 characters (e.g. SSW, SWxS, etc.)

712					 On the Form 720, the wind direction 							      (WNDDEG) is reported in whole degrees (0-							 360)

20  WI     	  wind speed indic.  Set to 5; Beaufort force

21  W      711	  wind speed         Convert to tenths of meters per second 							 according to Table K5-5, page K29 COADS                                          Release 1, Reference Manual

22  VI     	  vis. indic.        Set to missing (no visibility reported)

23  VV     	  visibility         Set to missing (none reported)

24 WW	 667  			      The 700 series uses Beaufort weather codes

/668					 Convert according to table 1 (not yet 			/669                      developed). Then translate to LMR format

Table 1

Beaufort weather     CLCOM conversion           LMR conversion

symbols                codes                      codes

B (blue sky)			 0					00

/clear/good

/fair/none/blue

C (cloudy)			 2					03

D (Drizzle)			51					51

E (wet without rain)	14					21

F (fog)				12					47

G (gloomy weather)		16					16

H (hail)				88					89

K (smoke)				 4					04

L (lightning)			13					13

M (mist)				10					10

O (overcast)			 3					03

P (passing showers)		81					81

Q (squalls, rough)		97					18

R (rain)				61					61

S (snow)				71					71

T (thunder)			17					17

U (ugly weather)		18					07

V (unusual visibility-	 0					00

exceptionally clear)

W (dew)				11					11

X (hoar frost)			48					48

Z (haze)				 5					05

Fog (written out)		45					45

Funnel Cloud			19					19

Note: All these symbols can be used in combination (up to three).  Whenever,

this occurs always select the higher value based on the LMR code numbers

(e.g. OPQ = 81, CRZ = 61, CR = 61, CPH = 89, and OC = 03) with the following exceptions:

Any non-precipitation event grouped with thunder should be coded as 17 unless grouped with a funnel cloud which should be coded as a 19.

BC (partly cloudy) = 02

Any combination containing thunder (T = 17) and P(passing showers, R(rain), or S (snow)  should be coded as a 95 (thunderstorm no hail). Any combination with T(thunder) and H (hail) should be coded as a 96 (thunderstorm with hail), for example:

RT = 95

ST = 95

HT = 96  any combination with HT should = 96

RST = 95  and any combination (excluding H) with RT and ST = 95

CRT = 95

PRT = 95

CST = 95

PST = 95

The original keyed Beaufort weather values (or CLICOM conversion codes) will be placed in the supplemental atachment.

25  W1           past weather 	 None reported in the 700 form series.

Assign as missing.

26  W2     	  2nd past weather   Assign as missing. None reported.

27  SLP    707	  sea level pressure Convert corrected barometer report in 			/607                      inches or millimeters to millibars using  		/708			           the conversion of 1 inch = 33.8639 			/608                      millibars or 1 millimeter = 1.333224 			                          millibars. If the corrected value is not 		                           available then the correction must be made                                       using the proper procedures based on                                             whether it is an aneroid or mercurial                                            barometer.  Translate into LMR.

704/604                    Attached thermometers (degrees F/C                                              respectively)

28  T1     701	  temp. indic.       Set to 6; whole degree Fahrenheit

29  AT     701	  air temp.          Convert from Fahrenheit to Celsius and          	                               translate to LMR.  C = (F-32)/1.8

30  WBT    702	  wet bulb temp.     Convert from Fahrenheit to Celsius and                                           translate to LMR.

31  DPT    	  dew point temp.    Not reported; assign as missing

32  SST    744	  sea surface temp.  Convert from Fahrenheit to Celsius;                                              translate to LMR

33  SI     	  SST method indic.  None of the 700 series contains information

on the SST method (bucket, intake, etc.) of 							 collection.  For those reporting a SST set

SI = 8 (unknown method), otherwise set to 							 missing.

34  N	 714	  total cloud amt.	 The total cloud amounts in the 700 series

were all reported in tenths and were never

contaminated by the 0-9 radio code as

happened to the 500 form series. Therefore,                                       convert the tenths to eights using table 2                                       below.  All cloud amount values are to be                                       preserved by storing the original value in

the supplemental attachment.  CLICOM                                            converted a dash (-) to an eleven (11).

Table 2

Converting cloud amounts in tenths to eights

tenths	        eights

0			0

1			1

2 or 3		2

4			3

5			4

6			5

7 or 8		6

9			7

10			8

Obscured		9  (not used)

11 (=dash)     blank

35  NH     	  lower cloud amt. 	 Never reported in the 700 series.

Assign as missing

36  CL     620	  low cloud type	 First cloud type does not necessarily 	                                           conform to a low cloud type.  Move to the                                        supplemental attachment and assign as                                           missing in the LMR format.

37  HI     	  cloud ht. indic.   Assign height indicator as missing, because 							 cloud heights were not reported in the form 							 700 series.

38  H      	  cloud height       Assign as missing; not reported

39  CM     621	  middle cloud type  Second cloud type does not necessarily                                            conform to a middle cloud type. If present                                       move to supplemental attachment and assign                                      as missing in the LMR format.

40  CH     625	  high cloud type    Third cloud type does not necessarily                                  	    	 conform to a high cloud type. If present                                          move to supplemental attachment and assign                                      as missing in the LMR format.

41  WD     	  wave direction     Assign as missing; place sea direction

in supplemental attachment because the only                                      other sea information reported is the sea                                        state which does not translate well to the                                       modern code.

42  WP     	  wave period        Assign as missing; not reported

43  WH     	  wave height        Assign as missing; not reported.

Place sea state in supplemental

attachment

44  SD     	  swell direction    Assign as missing; not reported

45  SP     	  swell period       Assign as missing, not reported

46  SH     	  swell height       Assign as missing,  not reported

47  C1     	  country code       Assign as missing, not applicable

48  C2     	  2nd country code   Assign as missing, not applicable

49  SC     	  ship course        Assign as misssing, not reported

50  SS     	  ship speed         Assign as missing, not reported

51  A      	  bar. tendency      Assign as missing, not reported

52  PPP    	  amt. of SLP change Assign as missing, not reported

EXCEPTION: Place 2 hour pressure change 							 reported on form 703 in the supplemental

attachment (element # 28) and the direction                  		 	   	 of change in element # 27 (characteristic                       		   	      of change).  Then convert to tenths of                             	           millibars (form 703) and multiply by 3/2 to                                      produce an estimated 3hr PPP to be placed                                        in element 52 (LMR).  For the LMR bar.                                           tendency, convert `+' to `2'(increasing                                         steadily or unsteadily); `-' to `7' 							     (decreasing steadily or unsteadily) and a                                         +0000 or -0000 to a `4' (steady, pressure                                       the same as 3 hours before).

53  IS     	  ice accretion      Assign as missing; not reported

54  ES     	  ice thickness      Assign as missing; not reported

55  RS     	  ice accretion rate Assign as missing; not reported

56  II     	  ID indic.          Set to 10; composite information

57-

64  ID(8)  	  ID/call sign       Assign 8 digit identifier as call sign.

Note: first two characters (57 & 58)

represent country of registry

(see list below) and the next six

digits (59-64) represent a single voyage

each numbered sequentially as they were

digitized.  The 8 characters can be used

for track checking, but not for ship 							      identification.

Codes		Country

AS, OS		Austria

AU			Australia

BX			Belgium

CH			Chile

CI			China

CN			Canada

CU			Cuba

CZ			Czechoslovakia

DD, DL		Germany

DN			Denmark

DR			Dominican Republic

FI			Finland

FR			France

GR			Greece

HK			Hong Kong

HO			Honduras

IY			Italy

JP			Japan

MX			Mexico

NK			Nicaragua

NL			Netherlands

NO			Norway

PH			Philippines

PL			Poland

PM			Panama

PO			Portugal

RA, RS		USSR

SN			Sweden

SP			Spain

SW			Switzerland

TH			Thailand

TU			Turkey

UK			United Kingdom

US			United State of America

YG 			Yugoslavia

65  OS     	  observation source Assign as missing

66  OP     	  observation plat.  Assign as missing

67  T2     	  2nd temp. indic.   Assign as missing; Wet bulb temperature

is reported on form 600 series.

68  IX     	  stn/weather indic. Assign as missing; not applicable

69  WX     	  wave per. indic.   Assign as missing; not applicable

70  SX     	  swell per. indic.  Assign as missing; not applicable

71  IRD    	  IMM receipt date   Assign as missing; not applicable

72  A6     	  allowance 6 flag   Assign as missing; not applicable

73  CK     	  checksum           Perform as outline in LMR documentation

74  AC     	  attachment count   Assign as described in LMR documentation

Irregular section to follow:

75  AL     	  attachment length

76  AID    	  attachment ID

77  AD     	  attachment data

Supplemental Attachment

Field	Element	Positions		Description

No.       No.

1				  1-3		Form Type (e.g. 500, 601, 721, etc.)

2		694		    4		Day of the week (GMT)

3		693		  5-6		Greenwich Mean Time (00-23)

4		649		    7		Octant ( 0-3; 5-8; missing = space)

5		711		  8-9		Beaufort Force (0-12; a dash = 13 or -)

6		610 		   10 		Visibility (0-9)

7		666		 11-12      	Present weather (00-99)

8		665		    13		Past Weather (0-9; a dash = 11 or -)

9		667		 14-19		Beaufort weather ( as initially coded                  /668                      {alpha characters} or converted by

/669                      CLICOM {numeric code values}

10		606		 20-21		Barometer as coded (PP; 00-99)

11		707		 22-25		Barometer as read (inches to 				    /607                      hundreths or millimeters to tenths)

Note: The instrument correction

in the header record is not always in

the same units as the "barometer as read"

units.

12				 26-29		Attached Thermometer

704				     Fahrenheit (whole degrees)

/604					Celsius (to tenths of a degree)

Note: It is not always the case that if the

barometer is reported in inches that the 							attached thermometer is in Fahrenheit or if

in millimeters the temperature is in 								Celsius. There is occasionally a mixture.

13		708		 30-33		Barometer as corrected (in. to hundreths)

/608                      Barometer as corrected (mm to tenths)

If both are reported (608/708) select

the one that has the units that agrees with

field # 9 (barometer as read), otherwise,                                         convert the one that is available to agree.

14		   		    34		Barometer type

15				 35-39		Barometer correction

16 		   		    40		Air Temperature Indicator (S = stationary 						     thermometer; P=psychrometer; O= other)

17		714		 41-42		N, total cloud amount (as recorded in tenths 							or code)  (0-10; a dash = 11 or -)

18		716		 43-44		Nh. Low cloud amount (as recorded in tenths 							or code)  (0-10; a dash = 11 or -)

19		620		 45-48		Cloud Type

e.g. FT511/FT601/FT701 - Cloud type 1

FT522/FT523 - high cloud type

20		615		 49-52		Cloud movement (direction)

/640					direction of upper (high) cloud movement

21		621		 53-56		Cloud Type

e.g. FT511/FT601/FT701 - Cloud type 2

FT522/FT523 - middle cloud type

22		641		 57-60		Direction of middle cloud movement

23		625		 61-64		Cloud Type

e.g. FT511/FT601/FT701 - Cloud type 3

FT522/FT523 - low cloud type

24		645		 65-68		Direction of low cloud movement

e.g. FT522/FT523/etc.

25		754		 69-71		State of Sea (alpha or numeric)

/657                      Note: there is a problem between the  						          conversion codes and the non-converted codes 							in the range of 1-10 since they have 							     different meanings, yet appear the same.

26		807        72-75     	Sea (wind wave) Direction (right justify - 8           /655                       pt scale; coded 0-9 or coded using alpha                                        representation; a dash = 11 or -)

27		755        76-79     	Swell Direction (right justify - 8 pt scale;  	    /803					coded 0-9 or coded using alpha                                                   representation; a dash = 11 or -)

28	    	757        80-82      	Character of swell (same as state of sea)

/804

29		605           83		Temperature difference (air-water) in

code (0-9)

Note: observer often entered

actual difference instead of

code. This can generally be checked

since both air and sea temperatures are

part of the basic record.

30		663		    84		Characteristic of change of the barometer

/805                      (a) in 3 hours (0-9; a dash = 11 or -)

Note: exception for form 703, a plus or

minus will be placed in this field to

indicate the direction of the 2 hour

pressure change reported in element 28

below (pp).

31		664		 85-86     	(pp) Barometer change in two digit code 	    /806					(00-99)

Note: exception for form 703, place the

two hour pressure change in this field

(hundreths of inches)

32				 87-112		Ship Name (26 characters allocated)